FAST AMP-Test board Manufacturer’s Instructions

University of Pennsylvania – Dept Physics

BOARD NAME: FAST AMP-TESTBOARD 2017

DATE: September 2017

CONTACTS:

Godwin Mayers: mayers@hep.upenn.edu

(215) 898-6809

Mitch Newcomer mitch@hep.upenn.edu

(215) 898-4456

MANUFACTURING NOTES

1. This is a 4 layer board

Layer order:

1. TOP

2. GND

3. POWER

4. BOTTOM

2. MATERIAL: FR4

3. BOARD THICKNESS: 0.062 inches

4. FINISH: Immersion Gold (MUST BE SUITABLE FOR WIRE BONDING)

5. COPPER THICKNESSES:

TOP: 1 oz

GND: 1oz

POWER: 1oz

BOTTOM: 1oz

6. EDGE PLATE boards on all sides to the extent possible

7. All holes are plated through unless otherwise noted